| # Application Sheet 1001 | # Revision 1.01 | # 11/09/2013 |
| --- | --- | --- |
| # How to colour you parts | # Created by | # K Dyke |

# Equipment needed

- Gloves

- Test Parts (for testing the dye and achieve the correct colour first)

- Saucepan

- Storage Jar Kilner Jar

- RIT Dye www.ritdye.com (purchase from eBay approx. £3-£5 for 30grams)

- Water from tap

- Heat source gas stove

- Something to stir and pick parts out (used a chop stick)

- Tray

- Kitchen towel

- Protective apron or lab coat

- Boil water and then turn down to simmer

- Add RIT dye

  - This is a mix used was 10grams to 500ml water but you can dilute or concentrate this to suit colour needed

- Turn off heat source

- Submerge part in RIT dye

- Leave from between 2-10 minutes depending on how deep you want the colour to be (use your test parts)

- Remove parts and place on kitchen towel allow to cool

- Rinse parts off in cold water

- Allow to dry (these parts were done using Navy Blue ones that look Black were left for 10 minuets +)

Note don’t have your dye too hot as it deforms the parts

- Store dye in Kilner jar for re use